# General License Examination – Memorization Sheet

General Class HF Frequency Privileges

10 meters	Generals have the same privileges as Extras; 28.020, 28.350, 28.550 MHz

12 meters	24.940 MHz

15 meters	21300 kHz

20 meters	14305 kHz Phone,

30 meters	10100 – 10150 kHz (CW and data only)

40 meters	7.250 MHz

75 meters      3900 kHz Phone

80 meters	3565 kHz

Maximum 1500 watts PEP, Except  200 watts PEP on 30 meters (10.140MHz), 100 watts for beacons

60 meter rules

5 authorized channels 2.8kHz wide with USB and 100 watts ERP maximum.  No interference to adjacent services and records must be kept if gain antenna is used.

RTTY/data near center of CW allocation (170Hz shift for amateur RTTY)

80m data   3570-3600	20m RTTY   14.070-14.100 MHz	20m PSK31   14.070

Maximum Symbol Rate for Packet, RTTY, or Data

Below 10 meters (28 MHz)	300 baud

10 Meter band	1200 baud

6 and 2 meters	19.6 kilobaud	Maximum 20Khz bandwidth

1.25 meters & 70 centimeter    56 kilobaud

General privileges can be used immediately with General CSCE by adding “/AG” to callsign on CW and “Temporary AG” on phone

Minimum Channel Separation

CW	150 – 500Hz	RTTY	250 – 500Hz	SSB	3 kHz

| Power Multipliers One S-Unit = 20dB = 100 fold power change 6dB = 4 fold power change 3dB = 2 fold power change 1dB loss = -20.5% Propagation A-index – Long term geomagnetic stability | Data Modes PSK31 uses varicode RTTY uses 5 bit Baudot with 170Hz shift Sideband Operation Below 14 MHz use lower sideband (LSB) Above 14 MHz use upper sideband (USB) |
| --- | --- |

K-index – Short term geomagnetic stability

Solar Flux – Radio energy at 10.7cm

D layer absorbs

E layer maximum single hop distance 1200 miles at altitude of 70 miles

F2 layer maximum single hop distance 2500 miles

X-rays take 8 minutes to arrive, coronal mass ejections (CMEs) take 20-40 hours to arrive

A two tone linearity test uses two non-harmonically related audio tones

Q-Signals and Prosigns

| QRP – Low power operation, ~ 5 watts on HF QRS – Send more slowly QRQ – Send faster QSL – Acknowledge receipt | QRV – Ready to receive KN – Listening for specific station(s) CL – Closing station AR – End of message |
| --- | --- |

Wire Sizes

15 amp circuit requires 14 gauge wire - 20 amp circuit requires 12 gauge wire

Series/Parallel	Series	Parallel

| Resistors/Inductors | Add (R1+R2+R3…) | Divide (1/ R1+R2+R3…) |
| --- | --- | --- |
| Capacitors | Divide (1/ C1+C2+C3…) | Add (C1+C2+C3…) |

Reactance

Increases with frequency in a coil, decreases with frequency in a capacitor

Components

Multiple-winding           NPN junction                      Field effect               Zener diode     Tapped inductor

transformer                  transistor                            transistor

Peak Envelope Power

PEP = [(0.707PEV)(0.707PEV)]/RL	Where:	PEV = Peak Voltage

RL = Resistive Load

Ohm's Law and Power Formulas

I2

R

E

I

I

R

E

P

P

E = Voltage in Volts

I = Current in Amperes

R = Resistance in Ohms

P = Power in Watts

Cover the value you need and divide or multiply the remaining values as appropriate

Examples:

E = I x R	I = E / R	R = E / I

P = I x E	I = P / E	E = P / I

P = I2 x R	R = P / I2	E2 = P * R

Antenna Lengths

| Dipole Length L (Ft) = 468 F (MHz) L is Length in Feet and F is Frequency in MHz | ¼λ Vertical L (Ft) = 234 F (MHz) Divide Full Wave Loop by 4 for one side of Quad Loop Divide Full Wave Loop by 3 for one side of Delta Loop |
| --- | --- |